Restraint for PVC pipe (AWWA C905 CI O.D.) at the bell shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. A backup ring shall be used behind the PVC bell. A restraint ring, incorporating a plurality of individually-actuating gripping surfaces, shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the bell ring and the gripping ring. The restraint devices shall be coated with MEGA-BOND. (For complete specifications on MEGA-BOND visit www.ebaa.com.) The restraint shall be the Series 2800, as manufactured by EBAA Iron, Inc., or approved equal.