Restraint of molded PVC fittings on IPS O.D. PVC pipe (ASTM D2241) shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated using MEGA-BOND®. (For complete specifications on MEGA-BOND visit www.ebaa.com.) Side clamp bolts shall be of SAE J429 Grade 5 material. A split, non-serrated ring shall be utilized on the fitting bell. A split, serrated ring shall be used to grip the pipe, and a sufficient number of bolts shall be used to connect the bell ring to the gripping ring. The restraint shall be the Series 7500, as manufactured by EBAA Iron, Inc., or approved equal.