Sample Specification HDPE

EBAA Series 15PF00, 2000PV, and 2100 restraint devices, 3-12 inch in

diameter, are designed to resist pull out forces based on the

maximum working pressure rating of the pipe. Forces experienced

due to expansion and contraction of the pipe require special

consideration.

EBAA products for HDPE are designed for underground, pressurized

fluid service. They are pressure rated to match the pipe SDR pressure

rating and derated as appropriate for service temperature. Maximum test

pressure is limited to pipe rated pressure.

Internal pipe wall stiffeners must be used when restraining HDPE.

The stiffeners must be sized to encompass the entire bearing length

of the restraint device. Pipe systems must be engineered to prevent

movement causing the fitting to slide or rotate on the pipe.