Mechanical Joint Restraint Sample Specifications

Restraint devices for mechanical joint fittings and appurtenances conforming to either ANSI/AWWA C111/A21.11 or ANSI/AWWA C153/A21.53, shall conform to the following:

# Design

- Restraint devices for nominal pipe sizes 3 inch through 54 inch shall consist of multiple gripping wedges incorporated into a follower gland meeting the applicable requirements of ANSI/AWWA C110/A21.10.

- The devices shall have a working pressure rating of 350 psi for 3-16 inch, 250 psi for 18-48 inch and 200 psi for the 54 inch size.  Ratings are for water pressure and must include a minimum safety factor of 2 to 1 in all sizes.

- All EBAA products intended for installation on ductile iron pipe are designed for and limited to use on ductile iron pipes that comply with the requirements of ANSI/AWWA C151/A21.51 and have a Brinell Hardness or equivalent measurement value that does not exceed 230BHN. These requirements apply to the entire pipe wall profile at all restraining wedge engagement points and to the full penetration depth of each restraining wedge.*

Material

- Gland body, wedges and wedge actuating components shall be cast from grade 65-45-12 ductile iron material in accordance with ASTM A536.

- Ductile iron gripping wedges shall be heat treated within a range of 370 to 470 BHN.

- Three (3) test bars shall be incrementally poured per production shift as per Underwriter’s Laboratory (U.L.) specifications and ASTM A536. Testing for tensile, yield and elongation shall be done in accordance with ASTM E8.

- Chemical and nodularity tests shall be performed as recommended by the Ductile Iron Society, on a per ladle basis.

# Traceability

# An identification number consisting of year, day, plant and shift (YYDDD)(plant designation)(Shift number), shall be cast into each gland body.

# All physical and chemical test results shall be recorded such that they can be accessed via the identification number on the casting.  These Material Traceability Records (MTR’s) are to be made available, in hard copy, to the purchaser that requests such documentation and submits his gland body identification number.

# Production pieces that are too small to accommodate individual numbering, such as fasteners and wedges, shall be controlled in segregate inventory until such time as all quality control tests are passed.  These component parts may then be released to a general inventory for final assembly and packaging.

# All components shall be manufactured and assembled in the United States.  The purchaser shall, with reasonable notice, have the right to plant visitation at his/her expense.

Installation

- Mechanical joint restraint shall require conventional tools and installation procedures per AWWA C600, while retaining full mechanical joint deflection during assembly as well as allowing joint deflection after assembly.

- Proper actuation of the gripping wedges shall be ensured with torque limiting twist off nuts.

Approvals

- Restraint devices shall be Listed by Underwriters Laboratories (3” through 24” inch size) and Approved by Factory Mutual (3” through 12” inch size).

- Mechanical joint restraint shall be Megalug Series 1100 produced by EBAA Iron Inc. or approved equal.

*To learn more about this addendum, please visit: https://ebaa.com/spec/dip

MEGA-BOND® Restraint Coating System

Coating for restraint devices shall consist of the following:

All wedge assemblies and related parts shall be processed through a phosphate wash, rinse and drying operation prior to coating application. The coating shall consist of a minimum of two coats of liquid thermoset epoxy coating with heat cure to follow each coat.

All casting bodies shall be surface pretreated with a phosphate wash, rinse and sealer before drying. The coating shall be electrostatically applied and heat cured. The coating shall be a polyester based powder to provide corrosion, impact and UV resistance.

The coating system shall be MEGA-BOND by EBAA Iron, Inc. or approved equal. Requests for approved equal must submit coating material and process details for review prior to bid.

For more information regarding MEGA-BOND, refer to the MEGA-BOND brochure or visit www.ebaa.com.