Series 1100SDB Sample Specification

A mid span restraint anchor point on existing ductile iron pipe shall consist of the following: The restraint shall be split for ease of installation and manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated using MEGA-BOND®. (For complete specifications on MEGA-BOND visit www.ebaa.com). The split restraint ring, incorporating a plurality of individually-actuating gripping surfaces, shall be used to grip the pipe. The device shall have a minimum working pressure rating as shown in the product brochure. The restraint shall be the Series 1100SDB, as manufactured by EBAA Iron, Inc. or approved equal.

All EBAA products intended for installation on ductile iron pipe are designed for and limited to use on ductile iron pipes that comply with the requirements of ANSI/AWWA C151/A21.51 and have a Brinell Hardness or equivalent measurement value that does not exceed 230BHN. These requirements apply to the entire pipe wall profile at all restraining wedge engagement points and to the full penetration depth of each restraining wedge.*

*To learn more about this addendum, please visit: https://ebaa.com/spec/dip